A.V.KAVERI,

192512188.

EXPERIMENT – 4

MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER FOR AM

COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

PROGRAM:

clc; clear; close all;

% Parameters fs = 100000; t = 0:1/fs:0.02; fm = 500; % Message frequency fc = 10000; % Carrier frequency fLO = 14550; % Local Oscillator (IF = 4550 Hz)

% Message Signal m = cos(2*pi*fm*t); % Carrier Signal c = cos(2*pi*fc*t); % AM Signal am = (1 + m).*cos(2*pi*fc*t);

% Local Oscillator

LO = cos(2*pi*fLO*t); % Frequency Mixing mixed = am .* LO;

% IF Filter

[b,a] = butter(4,5000/(fs/2));

IF = filter(b,a,mixed); % Envelope Detection env = abs(hilbert(IF)); % Display

figure; subplot(4,1,1) plot(t,m) title('Message Signal') grid on figure; subplot(5,1,2) plot(t,c) title('Carrier Signal') grid on subplot(5,1,3) plot(t,am) title('AM Signal') grid on subplot(5,1,4) plot(t,IF) title('Intermediate Frequency (IF) Signal') grid on subplot(5,1,5) plot(t,env) title('Recovered Signal') grid on

OUTPUT: